DISCLAIMER – MAJOR PORTIONS OF THIS DOCUMENT HAVE BEEN RETYPED. IT IS HERE TO FAMIARIZE YOU WITH THE DATA IN THIS AD. REFER TO OFFICIAL SOURCES WHEN COMPLYING WITH THIS AD.

TO PREVENT DAMAGE TO FUEL QUANTITY INDICATORS AND THE FUEL TANK TRANSMITTERS DUE TO

OVERHEATING, CONTD.

70-15-02

CESSNA

Amendment 39-1036

Applicability:  Applies to Models 336 (Serial Numbers 336-0001 through 336-0195), 337, 337A, B, C and D (Serial Numbers 337-0001 through 337-1193), T337B, C and D (Serial Numbers 337-0526 through 337-1193) Airplanes.

Compliance: Required as indicated, unless already accomplished.

To prevent damage to fuel quantity indicators and the fuel tank transmitters due to overheating which can result in smoke emission in the cockpit and inaccurate fuel quantity indication, accomplish the following:

Within 50 hours' time in service after the effective date of this AD, install a cooling duct directing ram air over the fuel quantity indicators and perform a functional ground check of the fuel quantity system in accordance with the instructions contained in Cessna Service Letter No. ME70-24, dated July 7, 1970, or later FAA approved revision, or any other method approved by the Chief, Engineering & Manufacturing Branch, FAA, Central Region.

This amendment becomes effective July 21, 1970.